Notes

General Overview

- 3 Guidance Laws

  - Proportional Navigation

    - Watches the angle of the target. If the angle is constant but the range is closing, it’s on a collision course. If the angle starts drifting, PN commands a turn proportional to how fast that occurs. It reacts to rotation, nothing else.

  - Augmented Proportional Navigation

    - Same as PN, but it has a correction term for a target that may be accelerating sideways, not just drifting. PN is behind because it only accelerates after the rotation starts, but APN compensates for the lag in advance.

  - Zero Effort Miss

    - Completely different. Asks, if I stopped steering now and just coasted, how far would I miss by? Then projects both trajectories forward and measures miss distance, and steers to drop it to 0.

- Increasingly realistic conditions development

  - Idealized Target Motion

    - Constant velocity

    - Constant Acceleration

    - Weaving

  - Synthetic noise corrupted state estimation (Extended Kalman Filter)

  - Hardware in the loop gimbal (for individual runs to find mechanical error)

  - Monte Carlo statistical analysis

  - Real Camera/Stereo Captured target motion

System Architecture

- Guidance Layer

  - PN, APN, ZEM implemented from vector formulations of line-of-sight(LOS) rate, closing velocity, target acceleration), partially from Palumbo, Blauwkamp, and Lloyd’s Modern Homing Missile Guidance Theory and Techniques and Zarchan’s Tactical and Strategic Missile Guidance

  - Algebraic verification found that APN and ZEM formulas reduce to be identical when their Navigation constants are equal( N and N_ZEM)

- State Estimation Layer (EKF)

  - EKF estimates relative target position and velocity from noisy LOS angle measurements, following standard homing-loop EKF formulation

  - Filter consistency is checked via Normalized Estimation Error Squared (NEES) statistic, detailed in Hays and Fatemi’s Combat System Filter Engineering

  - EKF works on a loop of two steps, predict and update.

    - Predict) Based on physics and the last estimate, where should the target be now? Uncertainty grows here since theres a time gap since the last measurement.

    - Update) New noisy angle measurement arrives, blend it with the prediction weighted by how much you trust each(making a better estimate and shrinking uncertainty)

  - Called an EKF instead of a  regular KF because it refers to the fact that turning positions into angle is a nonlinear operation(uses trig). Plain Kalman filter math only works for linear relationships. EKF hands it by locally approximating the nonlinear relationship as linear at each time step using a Jacobian. It then runs the same predict/update math on the local approximation.

- Statistical Layer

  - Guidance-Law performance is largely determined in Monte Carlo simulations, hit probability with 95% Wilson Score Confidence intervals, miss-distance distribution, time to intercept, autopilot saturation fraction, termination-reason breakdowns, all stored

  - Used a Wilson Score Interval because a plain hit rate +- spread confidence interval breaks down when the true hit rate is near the extremes (0 or 100%). It can output negative confidence intervals, or those that go past 100.

  - Wilson interval doesn’t just add/subtract the margin, it solves for the range of true probabilities that’d produce observed result, given the sample size(uses statistics magic). This keeps it bound between 0 and 100 no matter how extreme the observed rate is.

- Recorded Motion Layer

  - Stereo Camera used to track real moving target via YOLO detection. Azimuth(AZ) and Elevation (EL) is computer from pixel position.

  - Range is estimated with classical stereo triangulation(difference between left and right detection, utilizing distance between them to make trig)

  - Resulting timestamp, bearing, and range are converted into a smooth 3D trajectory using a median of a small window of close samples.

  - That trajectory is then fed into the simulation as a non-synthetic target motion model

  - Cubic Spline is utilized through recorded points so clean velocity/acceleration is found

Key Findings

- APN and ZEM are the same at matched navigation constants

  - Established in classical guidance theory that APN and ZEM/Optimal Guidance coincide when both Navigation Constants are equal (standard N=3)

  - Project verified known result against my own use of the formulas. Both reduce to (N/t_go) * v_rel_perp + .5*N*a_t_perp where * denotes a dot product.

  - This was also validated empirically across Monte Carlo runs where N = N_zem.

  - Project confirms the equivalence holds in this specific vector usage, both symbolically and empirically, rather than assuming it transfers automatically from a textbook derivation

  - Practical Implication for experimental design shows that when doing a “PN vs APN vs ZEM” comparison, you need to have different navigation constants

- Guidance Law performance against real recorded motion can invert expected ranking from idealized-motion theory

  - Some early monte-carlo runs showed that PN got an 8% hit rate vs 3% for APN and ZEM, opposite of what’s expected considering APN has an acceleration compensation term that’d be expected to outperform PN for a maneuvering target. Diagnostic inspection showed APN’s autopilot saturation fraction was 0% (meaning it wasn’t control limitations causing this), while PN’s was 11.1%. I believe APN’s advantage relies on a clean target-acceleration signal, and the acceleration derived from real motion, even after smoothing, carries more noise than the idealized constant-acceleration or sinusoidal weaving models APN is normally validated against. Need to do more trials, but this seems to be like a leading result with these trials. It shows that a guidance law can be theoretically superior but practically inferor, a failure mode that is isn’t structurally visible to any study using smooth analytic maneuvering models.

- Literature gap exists this project’s recorded motion addresses

  - Recent GNC Monte Carlo literature found every guidance-law statistical evaluation found uses a synthetic closed-form target motion model – CV, CA, sin/weave, helix, or physics-derived thrust/gravity profiles.

  - Some papers directly state deterministic target-motion assumptions are a limitation of the field and respond with a more complex synthetic model rather than capture real motion.

  - I theorize that it may be because it’s easier to replicate findings for others if you can simply say “Use this equation under these circumstances”

- Engineering Process findings worth noting/big stuff that was troublesome along the way

  - Initially, the correct version of Python to match with YOLO wasn’t available for the computer to work on the GPU, was considering working with Linux subsystem to get around this. Fixed as the python/CUDA software finally caught up with the hardware of this laptop

  - Guidance was initially computing commands from ground-truth target state even when EKF/sensor-noise estimation was enabled

  - Raw per frame stereo range estimates showed large jumps frame-to-frame due to ordinary YOLO bounding box jitter interacting with small change at close range sensitivity. Fixed with median filtering prior to use in trajectory model

  - Real time gimbal doesn’t work, will end up just not using it

Possible Poster Framing/Wording

- Implemented 3 classical homing guidance laws under EKF based state estimation with sensor noise, using Monte Carlo simulation with Wilson-Score confidence intervals. I then introduced a target motion model derived from real stereo-camera-captured motion, addressing a gap in the reviewed papers, where guidance-law monte Carlo evaluations rely only on idealized parametric target models. Statistical Comparison against real motion reveals guidance-law performance differences not predicted by idealized motion theory. It should be noted that the real target motion used is from human body movements scaled to missile-relevant range and speed.